Garage Operating System (GOS v1.0)

Project: 5-Gallon Tote Wall Rack System

Owner: Stephen

Date: 2026-02-28

Status: Design Finalized – Ready for Build

# 1. Project Objective

Replace traditional upper cabinets with a wall-mounted 5-gallon tote rail system that:

- Organizes by task and system

- Reduces cognitive friction

- Supports frequent access

- Uses minimal material vs traditional shelving

- Scales modularly

Primary design philosophy:

Operational efficiency over furniture aesthetics

# 2. Structural System Overview

## Configuration

- Wall-mounted rail rack

- 3-stud-wide module

- 6 vertical levels

- 2 totes per level

- 12 totes per module

Recommended build:

- 2 modules

- Total capacity: 24 totes

- Expected usage: 16–18 active totes

- 6–8 expansion slots

## Dimensions

Tote:

- 16.2” W

- 12.1” D

- 8.6” H

Rail spacing:

- 9.25” vertical (bottom-of-rail to bottom-of-rail)

Rail depth:

- 12.5”

Vertical start:

- First level ~12” above floor

# 3. Lumber Cut List (Per 3-Stud Module)

## Vertical Spines

(3) 2x4 @ 76”

## Rails

(24) Rails @ 12.5”

Material:

- 2x4 ripped to 2.5” depth

- Final size: 1.5” thick × 2.5” tall × 12.5” deep

## Optional Back Stops

(6) 1x2 @ 32”

## Fasteners

- 3” structural screws (verticals to studs)

- 2.5” screws (rails to verticals)

- Optional construction adhesive

# 4. Taxonomy – Storage Architecture

Organized by:

B) Project Type

C) Tool Platform

D) Repair Kits

# Domain Structure

## 🟦 F — Furniture / Woodworking

F-01  TRIM ROUTER

F-02  ROUTER BITS

F-03  DRILL ACCESS

F-04  POCKET HOLE

F-05  GLUE-UP

F-06  SANDING

F-07  FINISHING

F-08  HARDWARE INST

F-09  JOINERY HAND

F-10  JIG HARDWARE

Expected Active: 8–10 totes

## 🟩 H — Home Systems

H-01  PLUMBING

H-02  ELECTRICAL

H-03  DRYWALL

H-04  PAINT TOUCHUP

H-05  GENERAL REPAIR

Expected Active: 5 totes

## 🟥 M — Mechanical

M-01  CAR MAINT

M-02  SMALL ENGINE

M-03  SEASONAL

Expected Active: 2–3 totes

# 5. Labeling Standard

## Large Front Label

- ALL CAPS

- Max 14 characters

- Printed (not handwritten)

- Centered

## Small ID Code

Bottom-right corner:

Example: F-05

## Lid Edge Label

System name repeated (small)

# 6. Color Coding System

Colored peel-and-stick dots

Placed top-left corner of lid

🟦 Blue = Furniture

🟩 Green = Home

🟥 Red = Mechanical

No secondary color coding unless system expands significantly.

# 7. Zoning Layout (2-Module Simulation)

## MODULE 1 — Furniture (Left Wall)

Top:

F-09 JOINERY HAND

F-10 JIG HARDWARE

Upper Mid:

F-07 FINISHING

F-08 HARDWARE INST

Eye Level:

F-01 TRIM ROUTER

F-05 GLUE-UP

Lower Mid:

F-02 ROUTER BITS

F-06 SANDING

Lower:

F-03 DRILL ACCESS

F-04 POCKET HOLE

Bottom:

Expansion

Expansion

## MODULE 2 — Home + Mechanical

Top:

M-03 SEASONAL

H-04 PAINT TOUCHUP

Upper Mid:

H-03 DRYWALL

M-02 SMALL ENGINE

Eye Level:

H-01 PLUMBING

H-02 ELECTRICAL

Lower Mid:

H-05 GENERAL REPAIR

M-01 CAR MAINT

Lower:

Expansion

Expansion

# 8. Cabinet Replacement Decision

Upper cabinets above miter station are no longer required if:

- Precision tools already live in drawer system

- Daily consumables are staged at workstation

- No desire for aesthetic display cabinetry

System is optimized for:

- Grab-and-go kits

- Task isolation

- Reduced search time

# 9. Design Principles

- One bin = one responsibility

- No “Misc” bin

- IDs never renumbered

- Leave 20–30% expansion capacity

- Frequency determines height placement

# 10. Future Expansion Options

- QR code on bins → digital inventory

- Audit checklist system

- Annual kit review cycle

- Add 3rd module if >80% capacity reached

# 11. Psychological Design Notes

System designed to:

- Reduce cognitive switching cost

- Support ADHD-friendly reset cycles

- Encourage complete task closure

- Minimize visual clutter

- Maintain operational clarity

# 12. Status

✔ Structural design finalized

✔ Taxonomy finalized

✔ Label standard finalized

✔ Zoning plan finalized

⬜ Build not yet executed